# Assignemet 1

## JalaSetu: Farm-Pond Siting and Catchment Analysis

Name: Aditya Yadav

Roll Number: 12340100

GitHub Repository: <https://github.com/Lazypilottt/Jalasetu>

Final Working Front-end URL:

[EDITABLE PLACEHOLDER: Paste the deployed frontend URL here before submission.]

### Report note

This report follows the supplied main.md report format. The screenshot

locations are intentionally left as editable placeholders so final browser

captures can be inserted without changing the report structure.

[SCREENSHOT PLACEHOLDER 1 — Insert the landing page and map-selection view here.]

---

# Introduction

Rural communities depend on short and uncertain monsoon seasons, while

surface runoff is lost when there are no suitably located storage

structures. Farm ponds are an effective rainwater-harvesting intervention:

they slow runoff, recharge local soil, and provide water for livestock and

irrigation. This assignment delivers JalaSetu, a geospatial-assisted web

application that turns contour lines into a terrain model, recommends a

pond location, estimates its contributing catchment and expected collected

water, and explains the result on an interactive map. The report covers

requirements, architecture, algorithms, implementation, CSD design choices,

evaluation, and limitations.

## Motivation

Manual pond siting requires interpreting many contour lines, spotting

depressions, estimating upstream drainage, and comparing earthwork options.

This is slow and error-prone when maps are coarse or when village staff do

not have GIS expertise. JalaSetu makes the first-pass assessment repeatable:

one contour upload and an optional map-selected land area produce ranked

sites, transparent metrics, and boundaries that can be checked in the field.

It is a decision-support tool, not a replacement for a surveyor or land

records.

## Scope of the Project

The system parses KML/KMZ contours, builds an adaptive local-UTM DEM,

computes slope/depression/TWI suitability, ranks candidate pond footprints,

allows the user to select a 250 m radius land area on the map, prioritizes

sites in that area, delineates its upstream catchment, estimates SCS-CN

storm runoff and pond storage, and overlays the selected area, recommended

pond, alternatives, and catchment. It does not verify ownership, obtain

parcel boundaries, query a live historical rainfall service, perform

structural embankment design, guarantee infiltration, or replace a detailed

hydrological and geotechnical survey.

# Problem Statement and Requirements

The functional requirements are implemented as follows.

::: {#tab:requirements}

Requirement                           Implemented in

------------------------------------- -------------------

Satellite imagery display             frontend/src/components/MapView.jsx Esri imagery layer

Contour map visualization              KML/KMZ parser, DEM pipeline, and Leaflet results map

Available-land identification         Map click selection with 250 m radius in MapView.jsx

Catchment area estimation             app/services/catchment_delineation.py

Historical rainfall query             Not implemented; design rainfall is an explicit user parameter

Runoff volume estimation              SCS-CN runoff calculation in catchment_delineation.py

Pond depth / storage recommendation   pond_site.py stage-storage curve and design depth

Combined overlay / results view       ResultsLayer.jsx, site and catchment panels

: Functional requirements and where they are implemented

:::

## Non-Functional Requirements

The target is a normal analysis response in under 30 seconds for the

provided contour dataset and a responsive map interaction below 100 ms.

The frontend must build as a static Vite bundle and work in current Chrome,

Firefox, Safari, and Edge; the backend is stateless and can run with

multiple Uvicorn workers. A deployment target of four API workers and

approximately 20 concurrent analyses is reasonable for this assignment,

with uploaded files bounded by deployment configuration. GeoJSON is kept

to the computed boundaries rather than returning raster arrays. Inputs are

validated by FastAPI, temporary uploads are removed in a finally block,

and production deployments should replace permissive development CORS and

add authentication and rate limiting.

# System Architecture and High-Level Design

The browser provides the contour file and selected map centre/radius to a

FastAPI service. The route offloads the CPU-heavy analysis to a worker

thread, keeping the event loop available for health checks and concurrent

requests. The service runs the parser, DEM builder, terrain analysis, pond

siting, and catchment stages, then returns one JSON contract containing

metrics and GeoJSON. The present implementation has no persistent database

or external elevation/rainfall dependency: the uploaded contours are the

source data and the design rainfall/CN are explicit model inputs.

React + React-Leaflet map

| KML/KMZ + selected centre/radius

v

FastAPI /analyzeContour

parser -> DEM -> terrain suitability -> pond ranking

-> D8/pysheds catchment -> SCS-CN runoff

v

CatchmentResponse JSON + GeoJSON overlays

|

v

Site, catchment, water-volume panels and map layers

## Technology Stack

The backend uses Python 3.9, FastAPI, Pydantic, GeoPandas, Shapely,

Rasterio, SciPy, NumPy, and optional pysheds. The frontend uses React 18,

Vite, Axios, Leaflet 1.9, and React-Leaflet. GeoJSON is the interchange

format. No database or external rainfall/elevation API is queried in the

current offline-first version; this avoids rate-limit and availability

failures for a contour-only assignment and makes the demo reproducible.

Docker/Uvicorn deployment files are included for reproducible hosting.

# Methodology

The pipeline is a linear sequence over an adaptive raster. It releases

large intermediate arrays between stages and returns a bounded number of

candidate sites (max_candidate_sites, default 5), which limits response

size and memory pressure.

## Terrain and Elevation Analysis

kml_parser.py extracts line geometries and elevation attributes from KML

or KMZ. dem_builder.py samples contour vertices/segments, reprojects the

data to a local UTM CRS, and interpolates a regular DEM using linear

interpolation with a nearest-neighbour fallback outside the convex hull.

Resolution is data-driven unless overridden (the supplied dataset is

processed at metre-scale resolution). Terrain analysis derives gradient

slope, local depression/TPI measures, and topographic wetness index. A

weighted suitability score combines slope, depression, and TWI; only

contiguous regions above the threshold and minimum area are passed to

pond siting. This uses geospatial/numerical raster processing rather than

OpenCV because the input is vector contour data.

## Catchment Area Delineation

The recommended pond centroid is snapped to a high-accumulation cell within

the configured radius. When available, pysheds performs flow direction and

accumulation; the native fallback uses D8 neighbour routing and upstream

tracing. The resulting mask is polygonized and transformed to WGS84 for

the response. Catchment area is computed in projected metres, and mean

slope/elevation relief are calculated over the mask. The model assumes a

static DEM, one pour point, and no culverts, roads, soil infiltration

variation, or engineered diversions.

## Rainfall Data Integration

No live historical rainfall API is queried. Instead, the user-configurable

24-hour design rainfall defaults to 100 mm and the agricultural runoff curve

number defaults to 75. The SCS-CN equation converts rainfall excess and

catchment area to estimated_runoff_volume_m3. This is deliberately

explicit in the UI/API so a village engineer can replace the defaults with

an approved local design storm.

# Implementation

## Backend and API Design

| Method / path | Purpose |
| --- | --- |
| `GET /health` | Service liveness check. |
| `POST /analyzeContour` | Upload KML/KMZ, optionally include selected latitude, longitude and radius, and run the complete pipeline. |
| `POST /catchment/analyzeContour` | Compatibility route for the same analysis endpoint. |
| `GET /analyzeContour/schema` | Return response schema and default parameters. |

The API has no authentication in this academic prototype. FastAPI validates

file type, XML/ZIP integrity, numeric ranges, and slope relationships.

Pipeline failures are returned as explicit status/message/processing-notes

responses; HTTP 400/422/500 are used for invalid upload, unprocessable

input, and unexpected server failures respectively.

## Frontend and Visualization

The React client uses Leaflet base layers for Street, Satellite, and Hybrid

views. Clicking the map places a centre marker and a dashed 250 m selection

circle. Clicking Analyze sends that area to the backend. The returned

recommended pond footprint is amber, alternatives are slate, the upstream

catchment is translucent blue, and each polygon has a metrics popup.

Panels show recommended latitude/longitude, footprint, slope, design storage,

catchment hectares, SCS runoff volume, rainfall/CN assumptions, and

feasibility notes. The production bundle was verified with npm run build.

[SCREENSHOT PLACEHOLDER 2]

Insert a screenshot of the completed frontend here showing the sidebar,

selected land area, recommended pond overlay, catchment boundary, and result

metrics.

## Database and Storage

The current application is intentionally stateless: contour files are

stored only in a temporary server file during a request and deleted after

completion; computed results are returned as JSON/GeoJSON. This is suitable

for a demonstrator and avoids retaining land data. A production extension

would use PostGIS for villages, parcels, cached DEM metadata, rainfall

observations, and versioned analysis results, with spatial indexes on

geometry and a request/job identifier.

# CSD Themes and Topics Applied in the Project

| CSD Theme/Topic | Where used in the project | Justification / design rationale |
| --- | --- | --- |
| API Design (REST) | FastAPI `/health`, `/analyzeContour`, `/findCatchment` | A typed JSON contract keeps browser and pipeline independently testable. |
| Load Balancing | Uvicorn worker deployment configuration | Stateless requests can be distributed across workers for concurrent village queries. |
| Caching | Not implemented in the prototype | Avoids stale terrain results; production can cache immutable contour hashes and rainfall responses. |
| Database Indexing / Query Optimization | Bounded candidate count and projected-area calculations | Limits response size; PostGIS indexes are a production extension, not claimed here. |
| Concurrency / Asynchronous Processing | Async FastAPI route with `run_in_threadpool` | CPU-heavy raster work does not block the event loop, so health checks and other requests can proceed. |
| Microservices vs. Monolith | Modular monolith in `app/services/` | Shared in-memory DEM state makes one request cheaper and simpler than network hops. |
| Design Patterns | Pipeline orchestration, service classes, fallback strategy | Each stage has one responsibility and hydrology degrades explicitly to native D8. |
| Containerization / Deployment | Dockerfiles and Render/Uvicorn configuration | Reproduces Python and frontend deployment environments. |
| Error Handling and Resilience | Input validation, status values, processing notes, cleanup | Invalid files and missing optional hydrology support surface actionable results. |
| Algorithms and Complexity | Raster gradients, connected components, D8 upstream traversal | Work scales with raster cells; candidate and response counts are capped. |
| Testing Strategy | Unit, API, end-to-end, and distributed tests in `tests/` | Covers parser, DEM, terrain, siting, routes, and complete response shape. |
| Version Control / CI-CD | Git repository with separated `app`, `frontend`, `distributed`, and `tests` | Keeps deployable surfaces and regression tests reviewable. |

The most significant choices are the modular pipeline and explicit

fallbacks. Terrain and hydrology are computationally different concerns,

so their services can be optimized independently while the orchestration

keeps one request easy to reason about. The selected-area feature is

deliberately a bounded circle rather than an unconstrained polygon drawing

tool: it is fast, requires no extra drawing dependency, and prevents huge

requests while still letting a user target a parcel neighbourhood.

# Results and Evaluation

For the supplied contours_1m.kml, the representative run produced a

successful response with a recommended pond location and ranked alternative

sites. The no-upload map-only workflow also runs against the bundled contour

dataset when a complete selected area is provided. Exact values are

data-dependent and are returned by the API rather than hard-coded: the

reportable fields are recommended_site.latitude,

recommended_site.longitude, recommended_site.area_m2,

recommended_site.storage_capacity_m3, catchment.area_hectares, and

catchment.estimated_runoff_volume_m3. The map makes all three requested

outputs inspectable: the pond location/footprint, catchment boundary/area,

and expected water volume (storage and storm runoff) are shown in panels and

popups. The selected-area response also records centre, radius, and area.

[SCREENSHOT PLACEHOLDER 3]

Insert a screenshot of the map after analysis. The screenshot should show

the suggested pond location/footprint, catchment boundary, selected area,

and the visible water-volume/catchment summary.

## Performance

The complete backend regression suite completed 34 tests in approximately

15.5 seconds in the development environment. The API-route regression tests,

including the selected-area fallback, completed 8 tests successfully. The

Vite production build completed successfully in approximately 1 second and

generated a JavaScript bundle of approximately 411 kB (126 kB gzip) and a

CSS bundle of approximately 46 kB (12 kB gzip). These figures include test

and build startup overhead and are not a production load benchmark. Raster

cost is approximately linear in DEM cell count; for stress scaling, use

adaptive resolution, worker processes, upload limits, request timeouts, and

a job queue for very large files.

# Discussion and Limitations

The implementation is strong as a reproducible first-pass terrain

screening workflow: it uses projected geometry for area, emits standard

GeoJSON, exposes assumptions, and has a tested fallback when pysheds is not

available. Accuracy remains limited by contour spacing/interpolation,

unknown soil and land-cover parameters, the single design storm, and the

absence of parcel ownership and field survey data. The selected map circle

is a planning area, not a legal parcel. A production release should add

authenticated users, PostGIS, rainfall history with provenance, DEM quality

checks, background jobs, rate limiting, observability, and field validation.

# AI Tool Usage Declaration

AI tools, including GitHub Copilot, were used for repository inspection,

implementation assistance, debugging, frontend integration, and report

editing. The team reviewed the generated changes, understood the pipeline

and API contract, and validated the final implementation with the frontend

production build and targeted backend tests. AI assistance did not replace

engineering judgement, field validation, or responsibility for the submitted

design and results.

# Source Code and Repository

The source repository is

https://github.com/Lazypilottt/Jalasetu. The root contains the FastAPI

application in app/, the React/Vite client in frontend/, optional

distributed-stage demonstrations in distributed/, regression tests in

tests/, deployment files, the supplied contours_1m.kml, and this report

template filled with the implemented system details.

## Submission Details

Student: Aditya Yadav

Roll Number: 12340100

Assignment: Assignemet 1

Repository: <https://github.com/Lazypilottt/Jalasetu>

Final Working Front-end URL:

[EDITABLE PLACEHOLDER: Add the final deployed frontend URL.]

# Appendix: Reproducibility and Operations

Start the backend with uvicorn app.main:app --host 0.0.0.0 --port 8000.

Start the frontend with cd frontend && npm run dev.

Set VITE_API_BASE_URL when the API is not at http://127.0.0.1:8000.

Upload contours_1m.kml, click the target area on the map, and press

Analyze. Inspect the amber pond, blue catchment, green selected area,

storage capacity, runoff volume, and processing notes.

Run backend regression checks with

python -m pytest tests/ and build checks with cd frontend && npm run build.

Replace the screenshot placeholders in this report with final browser

screenshots and add the deployed frontend URL in the submission-details

fields.